<H3C>dis interface brief

Brief information on interfaces in route mode:

Link: ADM - administratively down; Stby - standby

Protocol: (s) - spoofing

Interface Link Protocol Primary IP Description

InLoop0 UP UP(s) --

NULL0 UP UP(s) --

Vlan1 UP UP -- 公网网段(33~39)

Vlan100 UP UP -- 内网10网段(1~12)

Vlan192 UP UP -- 内网192_1网(13~24)

Vlan200 UP UP -- 管理192_10网(25~36)

Vlan300 UP UP -- 开心私网192_20网(37~44

Brief information on interfaces in bridge mode:

Link: ADM - administratively down; Stby - standby

Speed: (a) - auto

Duplex: (a)/A - auto; H - half; F - full

Type: A - access; T - trunk; H - hybrid

Interface Link Speed Duplex Type PVID Description

GE1/0/1 UP 1G(a) F(a) A 100 内网10.0.0.x 网段

GE1/0/2 UP 1G(a) F(a) A 100 内网10.0.0.x 网段

GE1/0/3 DOWN auto A A 100 内网10.0.0.x 网段

GE1/0/4 DOWN auto A A 100 内网10.0.0.x 网段

GE1/0/5 DOWN auto A A 100 内网10.0.0.x 网段

GE1/0/6 DOWN auto A A 100 内网10.0.0.x 网段

GE1/0/7 DOWN auto A A 100 内网10.0.0.x 网段

GE1/0/8 DOWN auto A A 100 内网10.0.0.x 网段

GE1/0/9 DOWN auto A A 100 内网10.0.0.x 网段

GE1/0/10 DOWN auto A A 100 内网10.0.0.x 网段

GE1/0/11 DOWN auto A A 100 内网10.0.0.x 网段

GE1/0/12 DOWN auto A A 100 内网10.0.0.x 网段

GE1/0/13 UP 1G(a) F(a) A 192 内网192.168.1.x网段

GE1/0/14 UP 1G(a) F(a) A 192 内网192.168.1.x网段

GE1/0/15 DOWN auto A A 192 内网192.168.1.x网段

GE1/0/16 DOWN auto A A 192 内网192.168.1.x网段

GE1/0/17 DOWN auto A A 192 内网192.168.1.x网段

GE1/0/18 DOWN auto A A 192 内网192.168.1.x网段

GE1/0/19 DOWN auto A A 192 内网192.168.1.x网段

GE1/0/20 DOWN auto A A 192 内网192.168.1.x网段

GE1/0/21 DOWN auto A A 192 内网192.168.1.x网段

GE1/0/22 DOWN auto A A 192 内网192.168.1.x网段

GE1/0/23 DOWN auto A A 192 内网192.168.1.x网段

GE1/0/24 DOWN auto A A 192 内网192.168.1.x网段

GE1/0/25 DOWN auto A A 200 管理网192.168.10.x网段

GE1/0/26 DOWN auto A A 200 管理网192.168.10.x网段

GE1/0/27 UP 1G(a) F(a) A 200 管理网192.168.10.x网段

GE1/0/28 UP 1G(a) F(a) A 200 管理网192.168.10.x网段

GE1/0/29 DOWN auto A A 200 管理网192.168.10.x网段

GE1/0/30 DOWN auto A A 200 管理网192.168.10.x网段

GE1/0/31 DOWN auto A A 200 管理网192.168.10.x网段

GE1/0/32 DOWN auto A A 200 管理网192.168.10.x网段

GE1/0/33 DOWN auto A A 200 管理网192.168.10.x网段

GE1/0/34 DOWN auto A A 200 管理网192.168.10.x网段

GE1/0/35 DOWN auto A A 200 管理网192.168.10.x网段

GE1/0/36 DOWN auto A A 200 管理网192.168.10.x网段

GE1/0/37 UP 1G(a) F(a) A 300 开心私网192.168.20.x网

GE1/0/38 UP 1G(a) F(a) A 300 开心私网192.168.20.x网

GE1/0/39 DOWN auto A A 300 开心私网192.168.20.x网

GE1/0/40 DOWN auto A A 300 开心私网192.168.20.x网

GE1/0/41 DOWN auto A A 300 开心私网192.168.20.x网

GE1/0/42 DOWN auto A A 300 开心私网192.168.20.x网

GE1/0/43 UP 1G(a) F(a) A 300 开心私网192.168.20.x网

GE1/0/44 DOWN auto A A 300 开心私网192.168.20.x网

GE1/0/45 DOWN auto A A 1 公网

GE1/0/46 DOWN auto A A 1 公网

GE1/0/47 DOWN auto A A 1 公网

GE1/0/48 UP 1G(a) F(a) T 192 交换机Trunk网络+

GE1/0/49 DOWN auto A A 1

GE1/0/50 DOWN auto A A 1

GE1/0/51 DOWN auto A A 1

GE1/0/52 DOWN auto A A 1

<H3C>dis cur

#

version 7.3.28, Release 3507P18

#

sysname H3C

#

irf mac-address persistent timer

irf auto-update enable

undo irf link-delay

irf member 1 priority 1

#

dns server 114.114.114.114

#

lldp global enable

#

password-recovery enable

pex working-mode switch slot 1

#

vlan 1

#

vlan 100

name 10内网网段

#

vlan 192

#

vlan 200

#

vlan 300

#

stp global enable

#

interface NULL0

#

interface Vlan-interface1

description 公网网段(33~39)

dhcp client identifier ascii 78ed252bcd8c-VLAN0001

ipv6 address dhcp-alloc

#

interface Vlan-interface100

description 内网10网段(1~12)

#

interface Vlan-interface192

description 内网192_1网(13~24)

#

interface Vlan-interface200

description 管理192_10网(25~36)

#

interface Vlan-interface300

description 开心私网192_20网(37~44)

#

interface GigabitEthernet1/0/1

description 内网10.0.0.x 网段

port access vlan 100

#

interface GigabitEthernet1/0/2

description 内网10.0.0.x 网段

port access vlan 100

#

interface GigabitEthernet1/0/3

description 内网10.0.0.x 网段

port access vlan 100

#

interface GigabitEthernet1/0/4

description 内网10.0.0.x 网段

port access vlan 100

#

interface GigabitEthernet1/0/5

description 内网10.0.0.x 网段

port access vlan 100

#

interface GigabitEthernet1/0/6

description 内网10.0.0.x 网段

port access vlan 100

#

interface GigabitEthernet1/0/7

description 内网10.0.0.x 网段

port access vlan 100

#

interface GigabitEthernet1/0/8

description 内网10.0.0.x 网段

port access vlan 100

#

interface GigabitEthernet1/0/9

description 内网10.0.0.x 网段

port access vlan 100

#

interface GigabitEthernet1/0/10

description 内网10.0.0.x 网段

port access vlan 100

#

interface GigabitEthernet1/0/11

description 内网10.0.0.x 网段

port access vlan 100

#

interface GigabitEthernet1/0/12

description 内网10.0.0.x 网段

port access vlan 100

#

interface GigabitEthernet1/0/13

description 内网192.168.1.x网段

port access vlan 192

#

interface GigabitEthernet1/0/14

description 内网192.168.1.x网段

port access vlan 192

#

interface GigabitEthernet1/0/15

description 内网192.168.1.x网段

port access vlan 192

#

interface GigabitEthernet1/0/16

description 内网192.168.1.x网段

port access vlan 192

#

interface GigabitEthernet1/0/17

description 内网192.168.1.x网段

port access vlan 192

#

interface GigabitEthernet1/0/18

description 内网192.168.1.x网段

port access vlan 192

#

interface GigabitEthernet1/0/19

description 内网192.168.1.x网段

port access vlan 192

#

interface GigabitEthernet1/0/20

description 内网192.168.1.x网段

port access vlan 192

#

interface GigabitEthernet1/0/21

description 内网192.168.1.x网段

port access vlan 192

#

interface GigabitEthernet1/0/22

description 内网192.168.1.x网段

port access vlan 192

#

interface GigabitEthernet1/0/23

description 内网192.168.1.x网段

port access vlan 192

#

interface GigabitEthernet1/0/24

description 内网192.168.1.x网段

port access vlan 192

#

interface GigabitEthernet1/0/25

description 管理网192.168.10.x网段

port access vlan 200

#

interface GigabitEthernet1/0/26

description 管理网192.168.10.x网段

port access vlan 200

#

interface GigabitEthernet1/0/27

description 管理网192.168.10.x网段

port access vlan 200

#

interface GigabitEthernet1/0/28

description 管理网192.168.10.x网段

port access vlan 200

#

interface GigabitEthernet1/0/29

description 管理网192.168.10.x网段

port access vlan 200

#

interface GigabitEthernet1/0/30

description 管理网192.168.10.x网段

port access vlan 200

#

interface GigabitEthernet1/0/31

description 管理网192.168.10.x网段

port access vlan 200

#

interface GigabitEthernet1/0/32

description 管理网192.168.10.x网段

port access vlan 200

#

interface GigabitEthernet1/0/33

description 管理网192.168.10.x网段

port access vlan 200

#

interface GigabitEthernet1/0/34

description 管理网192.168.10.x网段

port access vlan 200

#

interface GigabitEthernet1/0/35

description 管理网192.168.10.x网段

port access vlan 200

#

interface GigabitEthernet1/0/36

description 管理网192.168.10.x网段

port access vlan 200

#

interface GigabitEthernet1/0/37

description 开心私网192.168.20.x网段

port access vlan 300

#

interface GigabitEthernet1/0/38

description 开心私网192.168.20.x网段

port access vlan 300

#

interface GigabitEthernet1/0/39

description 开心私网192.168.20.x网段

port access vlan 300

#

interface GigabitEthernet1/0/40

description 开心私网192.168.20.x网段

port access vlan 300

#

interface GigabitEthernet1/0/41

description 开心私网192.168.20.x网段

port access vlan 300

#

interface GigabitEthernet1/0/42

description 开心私网192.168.20.x网段

port access vlan 300

#

interface GigabitEthernet1/0/43

description 开心私网192.168.20.x网段

port access vlan 300

#

interface GigabitEthernet1/0/44

description 开心私网192.168.20.x网段

port access vlan 300

#

interface GigabitEthernet1/0/45

description 公网

#

interface GigabitEthernet1/0/46

description 公网

#

interface GigabitEthernet1/0/47

description 公网

#

interface GigabitEthernet1/0/48

description 交换机Trunk网络+

port link-type trunk

port trunk permit vlan 1 10 100 192 200

port trunk pvid vlan 192

#

interface GigabitEthernet1/0/49

#

interface GigabitEthernet1/0/50

#

interface GigabitEthernet1/0/51

#

interface GigabitEthernet1/0/52

#

scheduler logfile size 16

#

line class aux

user-role network-admin

#

line class vty

user-role network-operator

#

line aux 0

user-role network-admin

#

line vty 0 63

user-role network-operator

#

arp user-ip-conflict record enable

#

radius scheme system

user-name-format without-domain

#

domain system

#

domain default enable system

#

role name level-0

description Predefined level-0 role

#

role name level-1

description Predefined level-1 role

#

role name level-2

description Predefined level-2 role

#

role name level-3

description Predefined level-3 role

#

role name level-4

description Predefined level-4 role

#

role name level-5

description Predefined level-5 role

#

role name level-6

description Predefined level-6 role

#

role name level-7

description Predefined level-7 role

#

role name level-8

description Predefined level-8 role

#

role name level-9

description Predefined level-9 role

#

role name level-10

description Predefined level-10 role

#

role name level-11

description Predefined level-11 role

#

role name level-12

description Predefined level-12 role

#

role name level-13

description Predefined level-13 role

#

role name level-14

description Predefined level-14 role

#

user-group system

#

local-user admin class manage

password hash $h$6$oXPFGI0c6s84coj4$e/9EvOgCB4zxc8aAZjGv3hp8MWtozZ5ZfJ52Pd+EZrzrcuAv61Bz7+zvbcQ8m0Ax3luJPTy7J8BhRqMwmXKODg==

service-type telnet http https

authorization-attribute user-role network-admin

password-control length 4

password-control composition type-number 1 type-length 1

undo password-control complexity user-name check

#

local-user clouduser class manage

password hash $h$6$ba9I1Ab3eQ23BEK/$dZZmMZeQpDhrX89d7qOBswzc484/FlduxNKyz56G5WuENO6wRs+ApWxL1Vqf6DYw6+6qcsMCeJoAgHT8Bg2vtA==

service-type http

authorization-attribute user-role network-admin

#

security-enhanced level 1

#

netconf soap http enable

#

ip http enable

ip https enable

#

smartmc tc enable

#

cloud-management server domain oasis.h3c.com